Anjoy Vision (ShenZhen) Information Technology Co.,Ltd.

# Product Specification Data Sheet

# Model：MC-J82H (AI ISP)

| Customer ACK | Review | Approval |
| --- | --- | --- |

| Update Date | HW Version | Notes |
| --- | --- | --- |
| 2026/07/22 | V1.0 | Initial Release |

Contact Us

Address: Room 610, 6th Floor, Block A, Nanfeng Building, Nanshan Yungu Innovation Industrial Park, No. 4093 Liuxian Road, Pingshan Community, Taoyuan Street, Nanshan District, Shenzhen, Guangdong Province, China

Website: www.anjvision.com

Phone：0755-86568600

- Product Image （Single board 42x42）

- Key Features

- Processor: Dual-core ARM Cortex-A7 1.2GHz SoC, 2GB DDR4, 16MB Flash ROM.

- Image Sensor: 1/1.8” 8MP large-format sensor with built-in ISP, delivering exceptional day and night vision performance.

- Video Compression: H.265 video encoding with adjustable multi-resolution frame rates and dynamic bitrate control, reducing network transmission and storage requirements by half.

- Image Optimization: Supports electronic shutter, automatic day/night switching, auto white balance, backlight compensation (BLC), highlight compensation (HLC), and face overexposure prevention.

- Advanced Image Processing: Supports Wide Dynamic Range (HWDR/DWDR), 2D/3D Digital Noise Reduction (DNR), image mirroring/flipping, image cropping, and anti-flicker.

- OSD & Privacy: Supports multi-line OSD text overlay, custom titles, and privacy mask settings.

- IR-CUT & Lighting: Supports multiple day/night control modes and lighting control modes.

- Smart Detection: Supports intelligent alarm functions with precise detection of human figures, vehicles, and non-motorized vehicles, as well as face detection and capture.

- Event Linkage: Supports event-triggered actions including snapshot/recording, audio-visual alarms, mobile push notifications, relay output, FTP upload, and Email alerts.

- False Alarm Filtering: Supports filtering of stationary targets to prevent false alarms, customizable arming schedules, and custom alarm audio upload.

- Remote Access: Supports remote live view and playback via mobile app or PC at any time, with two-way audio intercom functionality.

- Cloud Storage: Supports cloud storage services with encrypted data transmission and storage, ensuring evidence security even if the device is damaged.

- Browser Compatibility: Supports plugin-free HTML5 access across multiple browsers, with excellent compatibility with Windows, Linux, and Mac operating systems.

- PTZ & Lens Control: Supports PTZ control and motorized lens control, with transparent pass-through for various Pelco commands (cruising, tracking, reset, lens learning, etc.).

- Protocol: Supports ONVIF protocols, ensuring compatibility with most third-party NVRs and management platforms on the market.

- Network Protocols: HTTP(s), RTSP, RTMP, FTP, SMTP, DHCP, NTP, DST, P2P, Hik, NFS (Network Storage), etc.

- USB Host Expansion: Supports USB Host peripherals (USB WiFi adapters, USB storage devices, etc.).

- FPC Expansion: Supports a 16-Pin FPC full-function tail board extension (Micro-SD card slot, Reset button, speaker output, amplifier enable, alarm input, relay output).

## 3. Technical Specifications

| Model | MC-J82H |
| --- | --- |
| System Architecture | Embedded Linux, ARM 1.2GHz SoC, 1G DDR4, 16MB Flash ROM, AI-ISP |
| Image Sensor | 1/1.8” 8MP CMOS |
| Compression Standard | H.264 / H.265 / H.265+ |
| Shutter Speed | Auto, 1/10s – 1/10,000s |
| Digital Noise Reduction | Supports 2D/3D DNR |
| Wide Dynamic Range | Supports HWDR/DWDR, >100dB |
| Encoding Mode | H.264 / H.265 / H.265+ |
| Bitrates | Adjustable from 512 Kbps to 12 Mbps,VBR/CBR |
| Main Stream | 25fps@8MP(3840x2160)/(3072x2048)/30fps@5MP(2592x1944)/(2880x1620)/ 4MP(2560x1440)/3MP(2304x1296)/1080P/720P |
| Sub Stream | 30fps@720x480 /640x480 (VGA)/640x360(360P)/352x288(CIF) |
| Audio Encoding | Supports G.711U, G.711A, AAC; supports noise reduction and echo cancellation |
| Audio Interface | 1-channel Input / 1-channel Output, 1-channel speaker out |
| Photosensitive Mode | Image-sensor photosensitive / CDS photosensitive |
| Lighting Mode | 2-channel PWM output; supports Auto / Dual-light / IR / Full-color(Default) |
| OSD Overlay | Supports channel name, date/time, custom titles, and multi-line OSD overlay |
| Privacy Mask | Supports up to 4 areas |
| Smart Detection | Motion detection, human detection, vehicle detection, non-motorized vehicle detection (with static target filtering), face detection |
| Alarm Linkage | Supports arming linkage for audio-visual alarms, snapshot/recording, mobile push notifications, alarm output, border flashing, FTP and Email; Supports custom alarm audio upload |
| PTZ / Lens Control | Supports motorized lens control and PTZ control; supports transparent pass-through of preset commands |
| Reset Function | Supports hardware reset and software reset |
| Alarm Interface | 1-channel Input / 1-channel Relay Output (requires K10 extend board) |
| IR-CUT | Supports 1-channel IR-CUT port (typical value: 12V 40-60Ω) |
| Network Protocols | HTTP(s)/RTSP/RTMP/FTP/SMTP/DHCP/NTP/ DST/P2P/Hik/NFS |
| ONVIF Compatible | Yes, compatible with most of the third-part NVR |
| Multi-Browser Support | Compatible with Firefox, Chrome, Edge, Safari, 360 Browser, etc. (Windows / Linux / Mac) |
| P2P Cloud Service | Supports mobile remote P2P access and cloud storage |
| Network Port | 1x RJ-45, 10/100Mbps auto-negotiation |
| Expansion Features | Supports USB Host expansion (USB WiFi adapter, USB storage, etc.); Supports 16-pin FPC full-function board expansion;(K10: Micro-SD card slot, Reset button, speaker output, amplifier enable, alarm input, relay output) |
| Operating Conditions | Power Input: 12VDC ±10%; Temperature: -30°C to 60°C; Humidity: 10% to 90% RH |

4. Pin-out Definitions

Figure 1 ：Encoding Board

- Notes：

- The large red numbers near the inner side of the main board indicate the pin sequence; the detailed pin definitions are listed in the tables above.

- 2. The USB interface outputs 12VDC by default. Please use our proprietary expansion board or implement your own power conversion to supply peripherals.

| Tag | Port Functions | Pin | Pin Definitions | Description |
| --- | --- | --- | --- | --- |
| D1 | 8-Pin Network and Power | 1 | DC 12V+ | DC 12V+ (10V-15V) |
|  |  | 2 | GND | GND(negative) |
|  |  | 3 | RJ45_6 | Network_RX_RJ6 |
|  |  | 4 | RJ45_3 | Network_RX_RJ3 |
|  |  | 5 | RJ45_2 | Network_TX_RJ2 |
|  |  | 6 | RJ45_1 | Network_TX_RJ1 |
|  |  | 7 | RESET | Reset(Active Low) |
|  |  | 8 | Link_led+ | Link Active Indicator |
| D2 | 2-Pin Audio-in / Mic | 1 | AUDIO_IN | Audio-In(Mic/Line) |
|  |  | 2 | GND | GND |
| D3 | 2-Pin Audio-out | 1 | AUDIO_OUT | Audio-out(Line) |
|  |  | 2 | GND | GND |
| D4 | - Pin Speaker Out (8Ω 2W) | 1 | SPK_OUT+ | Speaker-out+(amplifier out) |
|  |  | 2 | SPK_OUT- | Speaker-out-(amplifier out) |
| D5 | 4-Pin Light Control | 1 | CDS_IN | CDS photosensitive (3.3V-12V) |
|  |  | 2 | GND | GND |
|  |  | 3 | PWM1 | IR Light (reserved) |
|  |  | 4 | PWM0 | Warm Light |
| D6 | 4-Pin USB Extended Port | 1 | GND | GND |
|  |  | 2 | USB_D+ | USB data+ |
|  |  | 3 | USB_D- | USB data- |
|  |  | 4 | 12V | 12VDC output |
| D7 | 6-Pin Serial Port (AF/PTZ Control) | 1 | RESET | Reset(Active Low) hold 5s |
|  |  | 2 | UART_TX | Auto-Focus TX（AF） |
|  |  | 3 | UART_RX | Auto-Focus RX（AF） |
|  |  | 4 | UART1_TX | NC (reserved) |
|  |  | 5 | UART1_RX | NC(reserved) |
|  |  | 6 | SPK_EN | Speaker Enable |
| D8 | 2-Pin IR-CUT Drive Port | 1 | IR-CUT | IR-CUT Drive A |
|  |  | 2 | IR-CUT | IR-CUT Drive B |
| D9 | 16-PIN FPC Extended Port | 1 | IO_OUT |  |
|  |  | 2 | IO_INPUT |  |
|  |  | 3 | SD_CD |  |
|  |  | 4 | SD_D1 |  |
|  |  | 5 | SD_D0 |  |
|  |  | 6 | GND |  |
|  |  | 7 | SD_CLK |  |
|  |  | 8 | GND |  |
|  |  | 9 | SD_CMD |  |
|  |  | 10 | SD_D3 |  |
|  |  | 11 | SD_D2 |  |
|  |  | 12 | VDD |  |
|  |  | 13 | KEY_RST |  |
|  |  | 14 | 3.3VDC |  |
|  |  | 15 | 3.3VDC |  |
|  |  | 16 | 3.3VDC |  |

- Optional Expansion Boards

| Name | Model | Function Descriptions |
| --- | --- | --- |
| Expansion Tail Board | K10 | 16-Pin FPC full-function tail board;(Expands Micro-SD card slot, Reset button, speaker output, amplifier enable, alarm input, relay output) |
| AF Board | H-AF | Supports 4x / 5x motorized AF lenses on the market |
| WiFi Board | MT-W3 | 2.4G WiFi6 Bluetooth |

Note: Expansion boards require dedicated firmware for support.

WeChat Contact